Anaerobic valorization of sewage sludge pretreated through hydrothermal carbonization: volatile fatty acids and biomethane production

Abstract

Hydrothermal carbonization (HTC) emerged as an effective technology for the treatment of various types of wet biomass and organic residues, including sewage sludge, offering the potential for sludge reduction and resource recovery. HTC pretreatment impact on downstream sludge fermentation is investigated. Results obtained at optimal conditions for HTC pretreatment (170 °C for 30 minutes) indicated that soluble carbon was significantly increased in the liquid fraction, enhancing feedstock availability for fermentation. Semi-continuous fermentation of HTC-treated sludge resulted in a stable process in which a mixed microbial community produced volatile fatty acids (VFAs) with longer chain acids content, acidification yield of 0.59 ± 0.05 g COD-VFA g-1 CODin and volumetric productivity of 1.6 ± 0.5 g COD-VFA L-1 d-1. Biomethane Potential tests evidenced high values for hydrochar. Overall, the HTC pretreatment enables improved conversion efficiencies, in the view of valorizing the liquid for VFA synthesis and the hydrochar for biomethane production.

Keywords: Acidogenic fermentation; Anaerobic digestion; Fermentative microbial community; Resource recovery; Thermochemical processes.

# Introduction

Sewage sludge (SS) produced in wastewater treatment plants (WWTP) is traditionally one of the main issues in plant operation, as its treatment and disposal represents up to 50-60% of the overall operating costs (Maragkaki et al., 2018). Sewage sludge is a complex heterogeneous matrix, composed of a large variety of organic compounds such as proteins, carbohydrates and lipids, inorganic compounds like nutrients and heavy metals, and pathogens (Kacprzak et al., 2017). In this view, resource recovery is fundamental to promote environmental and economic sustainability of sewage sludge management (Ding et al., 2021), being biological treatments, thermochemical conversion, and land application of stabilized biosolids among the main strategies conventionally applied (Bora et al., 2020).

Anaerobic digestion for biogas production was one of the first processes to be applied at the full-scale, allowing to reduce the amount of disposed sludge and to recover energy (Catenacci et al., 2020). Otherwise, the interest towards anaerobic fermentation designed for volatile fatty acids (VFAs) production emerged more recently in the view of carbon recovery. In this case, recovered VFAs can be directly applied within the WWTP itself as additional carbon source for promoting the biological removal of nitrogen (Grana et al., 2024) and phosphorous (Diaz et al., 2022). Furthermore, one of the most promising applications for recovered VFAs is the production of polyhydroxyalkanoates (PHA), biopolymers being studied mainly as substitutes of conventional fossil-based plastics (Marreiros et al., 2023).

Several sludge pretreatments were tested to enhance sludge hydrolysis and to increase the amount of organics accessible to microorganisms for fermentation (Liang et al., 2021). A promising alternative is represented by hydrothermal carbonization (HTC), a thermochemical process in which the feedstock is treated at high temperature (between 170 and 350 °C) and pressure (between 5 and 25 MPa) conditions for a reaction time varying from minutes to hours depending on the desired output (Chen et al., 2021).

Reaction mechanisms of HTC include hydrolysis, dehydration, decarboxylation, condensation, polymerization, and aromatization. These reactions occur both sequentially and in parallel, depending on the feedstock. HTC treatment produces minimal gaseous effluent, primarily composed of CO2 (> 90%), CO, traces of H2, CH4, and low-molecular-weight hydrocarbons, resulting in minor carbon losses (Merzari et al., 2020). During HTC Millard-type reactions develop, causing amino acids and sugars to react, producing poorly degradable compounds (Dwyer et al., 2008; Wang and Li, 2015). In recent years, the HTC technology has been primarily employed to produce hydrochar, a solid matrix that can be utilized as biofuel, soil amendment, or adsorbent (Libra et al., 2011). On the other hand, the liquid fraction resulting from HTC is a mixture of water-soluble organic macromolecules(Cao et al., 2017), in which a significant fraction (approximately 20-50%) of organic substances remains (Usman et al., 2020). Although this stream has been often indicated as a byproduct to be disposed of, valorizing alternatives were investigated, including anaerobic digestion for biogas production (Ahmed et al., 2021; Danso-Boateng et al., 2015; Merzari et al., 2020) and microalgae cultivation (Mantovani et al., 2022).

In the view of sewage sludge valorization, only few studies focused on the use of HTC as pretreatment for improved sewage sludge handling, with contradictory conclusions regarding the impact of the process on the sludge fermentability (Castro-Fernandez et al., 2023; Chen et al., 2021). This paper aims to clarify the benefits of applying HTC to a more sustainable management of sewage sludge. The novelty of the current research relies on analyzing potential valorization alternatives for both the liquid phase and the hydrochar within the WWTP. Primarily, biological oxygen demand and fermentation batch tests were conducted to gain a preliminary understanding of the most promising HTC pretreatment conditions for maximizing the conversion into VFAs, in line with existing literature findings. In agreement with Chen et al. (2021) and Ahmed et al. (2021), the HTC reactor was operated at 170 and 190 °C for 30, 60 and 150 min, being the pretreatment time counted after reaching the target temperature. Even if 180 °C is considered by most authors the initiation temperature for HTC,  Chen et al. (2021) have demonstrated that lower temperatures, such as 170°C, result in the most significant release of carbohydrates and proteins leading to a boost in VFAs production. This was not observed treating the sludge at higher temperatures, as a higher degree of substrate degradation occurred. Then, subsequent fermentation semi-continuous tests and biomethane potential (BMP) tests were carried out to compare the performance of the HTC-pretreated sludge under previously identified operative conditions in comparison with untreated waste sludge. Additionally, further analyses were conducted to assess the sludge dewaterability and to evaluate overall process mass balances.

# Materials and methods

## Sludge origin

Microbial inoculum and sewage sludge were sampled from Sesto San Giovanni WWTP (northern Italy, managed by Gruppo CAP, 100,000 population equivalent). The biological treatment consists of two lines: 30% of the flow is processed by an activated sludge system, while 70% undergoes biofiltration. The secondary sludge is directed to the primary settler and discharged alongside primary sludge as mixed sludge. Details of the sludge line, featuring a full-scale acidogenic fermenter, are reported in Supplementary materials.

## HTC sludge pretreatment

The HTC pretreatment was performed in a pressurized vessel equipped with an external jacket for temperature control (Berghof BHM-700), a mechanical stirrer (BRM-2), and a data acquisition unit (BTC-3000). For each HTC batch, the reactor was filled with 700 mL of mixed sludge. In agreement with Chen et al. (2021) and Ahmed et al. (2021), the HTC reactor was operated at mild temperatures, 170 and 190 °C, for 30, 60 and 150 min, being the pretreatment time counted after reaching the target temperature. The heating time to reach the target temperature was in the range of 50-60 minutes, while after the pretreatment the reactor was rapidly cooled down in a water-ice bath. The pretreatment outlet was then centrifuged at 4000 rpm for 30 minutes to separate approximately 100 mL of hydrochar and 600 mL of process liquid (HTCPL). HTCPL and hydrochar samples were stored at 4°C before fermentation tests (max storage time 3 days), and BMP tests (max storage time of few hours).

## Fermentation tests

Fermentation tests were performed under two different configurations, namely (i) batch and (ii) semi-continuous. All configurations were operated at 30 °C and magnetically stirred. As inoculum, fermented sludge from Sesto San Giovanni WWTP was used, while the mixed waste sludge used as feedstock for fermentation (with or without HTC pretreatment) was sampled after dynamic thickening.

## Batch configuration

The feedstock of the batch fermentation tests was the HTCPL obtained at 170 °C by different pretreatment times. The experimental setup consisted of 0.5-L glass bottles filled by half, sealed with perforable rubber stoppers and caps. To prevent overpressure in the headspace, a needle was inserted into the stopper and connected to a tube allowing fermentation gas to be released via a water seal. At the beginning of each test, the headspace was flushed with molecular nitrogen (N2) for approximately 10 minutes to guarantee anaerobic conditions. Tests were run for 5 days, during which 30 mL of liquid were collected daily and the headspace re-flushed with N2. Inoculum and mixed sludge were characterized by measuring total solids (TS), volatile solids (VS), pH, VFAs, soluble chemical oxygen demand (sCOD), total chemical oxygen demand (CODtot), ammonium ion (NH4+), and phosphate ion (PO43-). The sampled liquid was analyzed to characterize the fermentate composition in terms of pH, VFAs, sCOD, CODtot, NH4+, and PO43. Tests were run in duplicate.

## Semi-continuous configuration

After identifying the most promising operational conditions to produce the HTC pretreated sludge (170 °C applied for 30 min), its fermentation performance was compared with the untreated sludge under semi-continuous conditions in completely stirred tank reactor (CSTR). The experimental setup comprised two 1-L bottles operated in parallel and equipped with two ports allowing for: (1) feeding and discharge, (2) gas collection. The gas port was equipped with a sampling point, with gas flow meter (µflow, BPC instruments) and the gas was finally collected in a gas bag (details reported in Supplementary materials). Loading and discharge operations were carried out using 100-mL plastic syringes and exchanging a volume of 410 mL out of the total 700 mL every 3-4 days, so that the average hydraulic retention time (HRT) was 5.2 ± 1.1 days. During these operations, an N2 gas bag served as gas buffer to prevent air infiltration. The bottles fed on untreated waste sludge and HTCPL were named SC.US and SC.HTC, respectively.

Trials lasted for 60 days. In the case of SC.HTC, a 14-day batch acclimatization period was carried out to allow microbial adaptation, where the HRT was set three times higher than the target for the following continuous operation. The HRT corresponded to an input organic loading rate (OLR) of 9.5 ± 3.3 g CODin L-1 d-1 (7.4 ± 1.9 g VSin L-1 d-1) for SC.US and 4.0 ± 1.2 g CODin L-1 d-1 (2.9 ± 0.8 g VSin L-1 d-1) for SC.HTC. Three samples per week of feedstock and fermentate were collected and characterized in terms of TS, VS, volatile suspended solids (VSS), total suspended solids (TSS), pH, VFAs, sCOD, NH4+, and PO43-. The mixed microbial community was taxonomically characterized in the inoculum, in the untreated sludge (feedstock of SC.US), in the SC.US reactor outlet after 42 d of semi-continuous operation (end of the SC.US experiment) and in the SC.HTC reactor outlet after 27 and 56 d (approximately at half and end of the SC.HTC experiment).

## Analytical methods

## Chemical-physical characterization

TS, VS, TSS, and VSS contents were determined in duplicate in accordance with the Standard Method 2540 (APHA, 2005). pH was measured potentiometrically directly on liquid samples using a portable probe (HACH, HQ40d). sCOD, NH4+, NO3-, PO43- were measured using colorimetric method via HACH kits (LCK014, LCK302, LCK339, and LCK350) after centrifugation (Centurion Scientific, C2 series) and sample vacuum filtration on 0.45-μm pore size filters. Total COD was determined using the APAT/IRSA-CNR 5130 method. VFAs (acetic, propionic, iso-butyric, butyric, iso-valeric, and valeric) were monitored via the Standard Method 5560 (APHA, 2005), using a gas chromatograph (DANI Master GC) with flame ionization detection (FID 86/10). The compositional analysis of the fermentation gas was conducted through gas chromatography (DANI Master GC) with thermal conductivity detection (TCD 86/40). Metals and total phosphorus analysis was performed according to aqua regia digestion (UNI EN 13657:2004) followed by ICP-OES analysis (EPA, 2018).

## Biodegradation assessment

Biological oxygen demand (BOD) was characterized according to the Standard Method 5210 (APHA, 2005) on the HTCPL, while the BMP was assessed according to Catenacci et al. (2019) on the different HTC streams, and on the outlet of semi-continuous fermentation tests. The inoculum for the BMP tests was sampled from a full-scale anaerobic digester fed on waste sludge. Two sets of BMP tests, namely BMP1 and BMP2, were conducted during the experimentation, the first one was run in winter and the second in summer.

## Sludge solid-liquid separation tests

Sludge solid-liquid separability was assessed through the sludge volume index (SVI), the capillary suction time (CST, performed with Type 340B CST device), and the time-to-filtration (TTF, performed with 11 µm filter cut-off) methods, as reported in the Standard Methods (APHA, 2005).

## Microbiological characterization

Total DNA was extracted from 0.5 mL of sludge using the Fast DNA® Spin kit for soil (MP Biomedicals, USA) according to manufacturer instructions. The V5-V6 hypervariable regions of 16S rRNA gene for bacteria, and the V3 hypervariable region for archaea were PCR-amplified as described in Ferrentino et al. (2016) and the amplicons were sequenced by MiSeq Illumina (Illumina, Inc., San Diego, CA, USA) with a 2 x 300 bp paired-end protocol. Amplicon sequence variants (ASVs) were inferred using the DADA2 pipeline (Callahan et al., 2016), with trimming of the forward and reverse reads after 180 and 150 bases, respectively, and quality filtering with a maximum number of expected errors equal to 0.5 per read. The two reads were therefore merged and taxonomically assigned with the latest version of RDP Classifier (v. 2.14, August 2023).

## Data processing

The hydrolysis yield was calculated as the ratio between the solubilized COD and the amount of VS in the inlet, according to Eq. 1.

Hydrolysis yield=gsCOD,out-gsCOD,in gVS,in 				Equation 1

Where gsCOD,in and gsCOD,out are the mass of soluble COD (sCOD) of the inlet and outlet, and gVS,in is the mass of volatile solids of the inlet.

The acidification efficiency was determined as the ratio between VFAs (expressed in terms of COD) and sCOD in the fermentation outlet, according to Eq. 2.

Acidification efficiency=gCOD-VFA,outgsCOD,out 				Equation 2

Acidification yield was calculated as the ratio between the net VFA production and the total COD in the feed, as reported in Eq. 3.

Acidification yield=gVFA,out-gVFA,in gCOD,in 				Equation 3

Where gVFA,in and gVFA,out are the mass of VFAs of the inlet and outlet, and gCOD,in is the mass of COD of the inlet. Note that the acidification yield can be calculated for the fermentation process alone (acidification yield) or for the overall system, including both HTC and fermentation (overall fermentation yield).

Statistical t-test (with a significance level α equal to 0.05) was used to evaluate the significance of the observed difference for fermentation of the HTCPL and the US.

# Results and discussion

## HTC pretreatment

The parameter used to assess the efficiency of the HTC pretreatment was the specific release of sCOD and VFAs at the end of the HTC process with respect to the initial presence of such components in the sewage sludge, respectively 4.5 and 0.66 g L-1 for sCOD and VFAs. Experimental results are reported in Table 1, indicating that sCOD and VFA content increases with the increase in the process time at the operational temperature, as a longer reaction time leads to a greater carbon solubilization. Concerning the process temperature, HTC at 170 °C led to higher sCOD and VFA release compared to 190 °C. This is likely due to the higher degree of carbonization, decreasing the carbon available in the liquid form at higher temperature (Suryawan et al., 2024).  Thus, pretreating the sludge at 170 °C is, concerning the pretests performed, the most favorable choice for using it as a feedstock in a fermentation process. In addition, considering also the subsequent biological processes, the choice for milder temperature conditions is supported by the findings of Chen et al. (2021), that obtained higher acidification efficiency in converting a HTC pretreated sludge at 170 with respect to 190 °C. Previous experiments have evaluated the effect of thermal treatments on the different components of the sludge. Bougrier et al. (2008) measured an increase in soluble carbohydrate concentration of an order of magnitude up to 130 °C, and then decreased after 170 °C. Mottet et al. (2009) observed that carbohydrate solubilization increases up to 165 °C and then decreases between 165 and 220 °C suggesting that, at high temperatures, a fraction of the carbohydrates initially solubilized reacts with other carbohydrates or with a small portion of the proteins through Maillard reactions leading to the formation of melanoids, refractory to (Bougrier et al., 2008). The solubilization of sludge proteins shows a similar behavior  except that a decrease in solubility was observed at high temperatures (Bougrier et al., 2008; Mottet et al., 2009). As for anaerobic degradability, a stabilization or reduction of degradability was observed in various works by pretreating sludge above 170-190 °C. Bougrier et al. (2008) and Carrère et al. (2008) identified the limit as 190 °C, while Fdz-Polanco et al. (2008) and Mottet et al. (2009), reported this limit to be around 165-170 °C. In addition, it is worth nothing that if the hydrochar is the main product of the HTC process, which is not the case for this study, 170 °C is lower than average temperatures reported in literature studies.  As for the HTC process time, the sCOD specific release rate was 266 mg sCOD g-1 CODin h-1 for 30 minutes reaction and decreases to 191 mg sCOD g-1 CODin h-1 and 110 mg sCOD g-1 CODin h-1 for 60 minutes and 150 minutes, respectively. The slower COD solubilization observed with longer process times could be attributed to the combination of many factors. Initially, the easily soluble components might be rapidly released, resulting in a high sCOD solubilization rate. As the reaction progresses, the remaining organic matter becomes more recalcitrant, requiring more time and energy to solubilize. Thus, longer process times lead to a slower COD solubilization with a higher associated energy cost. Finally, considering the produced hydrochar volume and total solid content , an hydrochar yield of 65±15% was achieved (amount of the inlet solids being recovered as hydrochar).

## Batch fermentation tests

The HTCPL produced at 170 °C after 0, 30 and 60 min of HTC treatment was used in batch fermentation tests to assess the fermentation potential. The specific acidification yield of the batch fermentation tests was 38±7, 137, and 136±20 g COD-VFA g-1 CODin for 0, 30, and 60 minutes, respectively. Experimental results evidenced that the specific net VFAs production obtained after 30 and 60 minutes was very similar, with the 30-minute HTC pretreatment time being less energy demanding and, therefore, preferable. Based on these results, the HTC pretreatment at 170 °C for 30 minutes was selected as the most adequate to favor sludge fermentability while minimizing energy costs and this operational condition was applied in the following steps. As one of the most critical aspects of HTC is the formation of potentially toxic compounds, BOD was measured on sample both undiluted and 50% diluted. Indeed, even though toxic organics such as phenols and aromatic amines require more intensive conditions for their formation, some toxic organic compounds can form even at lower temperatures and pressures, as in the conditions used in this study. For instance, Garlapalli et al. (2016) reported the presence of furfural, HMF, and guaiacol in the HTC liquor produced by treating digestate at 180°C for 1 hour.

The BOD5/sCOD and BOD20/sCOD were, respectively, 56% and 67% for the undiluted HTCPL, and 59% and 75% for the diluted HTCPL. These results suggest that the HTCPL exhibits good aerobic biodegradability, indicating more promising results than what found by Danso-Boateng et al. (2015), that reported a BOD5/COD ratio of 41% and 44% for diluted (1:10) HTCPL obtained by treating primary sludge at 160 and 180 °C, respectively. Moreover, the diluted HTCPL showed a slightly higher BOD20/sCOD ratio (14.3% higher) than the undiluted sample, likely indicating that the dilution of toxic compounds in the HTCPL improves feedstock biodegradability.

## Semi-continuous fermentation tests

## Process monitoring

As reported in section 2.3, the untreated sludge and the HTCPL were used as feedstocks for semi-continuous fermentation tests, respectively for SC.US and SC.HTC (see Supplementary Materials for average composition of the feedstocks).

The COD and VFAs in the fermented liquor during the course of the experimentation are reported in Figure 1A for SC.US and SC.HTC. The sCOD and VFA in the SC.HTC suggest a stable process performance, possibly since the HTC pretreatment solubilizes most of the COD, making the organic carbon available in the reactor for assimilation by bacteria and conversion into VFA, so that the sCOD and VFA contents are mainly influenced by the sole fermentation process. Conversely, the process performance for the SC.US appears to be variable, as suggested by the variable sCOD value in the fermentate. Indeed, sCOD in SC.US was strongly influenced by both the hydrolysis of the particulate solids in the untreated sludge and the fermentation process. Regarding VFA, higher net productions were observed for SC.US compared to those measured in SC.HTC. This could be related to the high OLR in the SC.US due to the presence of particulate organics in the SC.US feedstock. The two reactors were indeed operated with the same HRT but significantly different OLR (9.5±3.3 and 4.0±1.2 g CODin L-1 d-1 for SC.US and SC.HTC, respectively). In accordance, the volumetric productivity resulted in 2.1±0.6 and 1.6±0.5 g COD-VFA L-1 d-1, respectively for SC.US and SC.HTC.

Figure 1B depicts the concentrations of nutrients (ammonium and phosphorus) in the fermentate. The average concentrations of nutrients in the feedstocks were 42±7 mg N-NH4 L-1 and 43±10 mg P-PO4 L-1 for untreated sludge and 168±12 mg N-NH4 L-1 and 37±5 mg P-PO4 L-1 for the HTCPL. Ammonium concentrations in both fermentates exhibit a variable trend due to the variability in the nitrogen content in the feedstock, but the ammoniacal nitrogen release is higher in SC.HTC due to a more pronounced protein hydrolysis. Thus, the fraction of VFAs produced from amino acids acidification is likely greater than SC.US. Phosphorus in SC.HTC remained relatively constant over time, with values lower than those measured in SC.US. Indeed, during the HTC pretreatment, most of the phosphorus is concentrated in the hydrochar (Boniardi et al., 2024), reducing its concentration in the HTCPL. In contrast, organic phosphorus is released by hydrolysis in SC.US, as highlighted by the increasing concentration of P-PO4 over time.

Regarding the pH, it increased during the batch SC.HTC acclimatization period until stabilizing between 5.6 and 6.1. In contrast, the pH in SC.US decreased from 5.5 during the first 4 days and later remained between 5.0 and 5.3, in agreement with the higher VFA and lower ammonium concentrations compared to the SC.HTC fermentate (see Supplementary Materials for pH trend in time). Gas production was continuously monitored and followed a linear trend for both reactors (see Supplementary materials). The gas production rate was 132 and 88 NmL d-1 respectively for SC.US and SC.HTC, indicating a lower biogas production for SC.HTC. In contrast, the average gas composition was very similar (2±4% H2, 62±7% CO2 and 36±8% CH4 for SC.US, 0±% H2, 61±9% CO2 and 39± 9% CH4 for SC.HTC). Hydrogen was detected only occasionally; however, as reported by Christopher et al. (2021), hydrogen poses relevant challenges regarding saturation processes, likely making its measurement in the gas phase not always indicative of its biological production. Overall, SC.HTC showed lower COD losses due to biogas production considering the lower gas production rate and the similar gas composition, as quantitatively detailed in section 3.6.

Figure 2 illustrates the VFA average composition in SC.US and SC.HTC after process stabilization, while its time trend is reported in Supplementary Materials. Depending on the feedstock, a different VFA composition was obtained. Acetic acid concentration is similar in both reactors, while the SC.US fermentate has a higher percentage of propionic acids. Long-chain acids, including butyric, valeric, caproic, and their isomers, are higher in SC.HTC (accounting for 45.4% of the total VFAs compared to 32.1% in SC.US). The statistical significance of such difference was demonstrated by means of t-test for propionic, iso-butyric, butyric, iso-valeric, valeric, and caproic acids, showing p-values always lower than 10-5. Conversely, p-values of acetic acid and ethanol were above the significance level (0.38 and 0.83, respectively). Therefore, the thermal hydrolysis resulting from HTC pretreatment promoted the conversion into longer chain VFAs, as confirmed by the different fermentative microbial communities as reported in section 3.3.3. As mentioned above, a higher conversion of amino acids could be one of the factors differentiating the two tests, leading to a different VFAs profile, since longer chain VFAs are primarily produced from proteins. If the fermented liquor is intended to be used as a feedstock for PHA production, the presence of longer chain VFAs is preferable (Marreiros et al., 2023; Matos et al., 2021), suggesting a valuable downstream application for to the HTCPL fermentate. Total solids in the fermented liquor remain constant from the end of the acclimatization period onward. In SC.US an average TS concentration of 37.9±3.7 g L-1 with an VS/TS ratio of 75±2% was measured, resulting in a limited decrease with respect to the inlet (average TS of 42.5±6.8 g L-1 and average VS of 33.6±5.2 g L-1). In contrast, thanks to the solid-liquid separation after the HTC pretreatment, in the SC.HTC fermenter TS averaged 8.9±0.8 g L-1, with an VS/TS ratio of 83±2% and a TSS concentration of 1.8±0.2 g L-1 with an VSS/TSS ratio of 85±12%. Considering that the SC.HTC was characterized by an average TSS content of 1.2±0.1 g L-1 and an average VSS content of 1.0±0.1 g L-1, the limited increase in the fermenter is likely due to cellular growth.

## Process performance indicators

As expected, the HTC pretreatment allowed achieving a more effective hydrolysis process. Indeed, hydrolysis efficiency was 0.51±0.06 g sCOD g-1 VSin and 0.36±0.06 g sCOD g-1 VSin with and without the HTC pretreatment, respectively. The acidification efficiency was 96±5% and 66±5% for SC.US and SC.HTC, respectively. This indicates that the carbon solubilized from the untreated sludge is entirely converted into VFAs, while a fraction of the thermally solubilized carbon is poorly fermentable or recalcitrant. This aspect is in accordance with literature studies (Dwyer et al., 2008; Wang and Li, 2015) and with the results of BOD assessment reported in section 3.1. Nonetheless, the HTCPL is well fermentable, with an efficiency of 0.59±0.05 g COD-VFA g-1 CODin, higher than 0.28±0.06 g COD-VFA g-1 CODin obtained by fermenting untreated sludge. This value is comparable with values reported by Chen et al. (2021), that observed efficiencies of 0.59 g COD-VFA g-1 CODin by fermenting HTCPL at 170 °C for 30 minutes. The latter was, however, obtained in a batch reactor after a reaction time of 9 d, likely promoting better conditions for VFA accumulation. As already commented, SC.HTC had a more stable behavior, with improved fermentation of the HTC pretreated feedstock compared to the untreated sludge. However, the overall acidification yield referred to the untreated sludge entering the HTC pretreatment (as explained in section 2.5) was 0.32±0.04 g COD-VFA g-1 VSin (0.22±0.03 g COD-VFA g-1 CODin), similar to 0.35±0.08 g COD-VFA g-1 VSin (0.24±0.06 g COD-VFA g-1 CODin) obtained in SC.US, which does not suggest any significant advantage in the HTC pretreatment when VFA production is the main target. However, other aspects are to be considered in process evaluation and comparison, including dewaterability and the management of the residual solids (see sections 3.4 and 3.5). Furthermore, the overall acidification efficiency was higher than what reported by Castro-Fernandez et al. (2023) that found a value of 0.17 g COD-VFA g-1 CODin.

## Microbial community analysis

The bacterial and archaeal communities were characterized in the fermented sludge from the full-scale fermenter used as inoculum in both reactors, and in the untreated waste sludge (Figure 3). Microbiological characterization showed highly biodiverse communities, which is generally an indication of the absence of strong selective pressures. Particularly, untreated sludge was dominated by members of the family Comamonadaceae, both uncharacterized (13.3%) and belonging to genus Acidovorax (8.8%). Comamonadaceae are well known for their huge metabolic diversity and are environmental bacteria typically found in many water and soil habitats (Willems, 2014). Members of this family, and particularly the genus Acidovorax, are typically aerobic, although some of them can also use nitrate as electron acceptor in anaerobic conditions. Instead, they are not known as fermenters (Du et al., 2022). In contrast, the bacterial community developed in the WWTP full-scale fermenter still contained high abundances of uncharacterized Comamonadaceae (8.8%) and of Acidovorax (4.6%) but was also co-dominated by members of the order Bacteroidales, both unclassified (11.4%) and belonging to genus Xylanibacter (6.6%). Bacteroidales are known to be mainly saccharolytic or peptidolytic, preferring complex polysaccharides and primarily producing acetate and succinate as fermentation products (Brown, 2014), while Xylanibacter can specifically ferment various sugars including xylan, xylose, pectin and carboxymethylcellulose (Hitch et al., 2022). Therefore, although a fraction of generalist, non-fermenting heterotrophic bacteria survived the fermentation process, the abundance of fermenters was greatly increased with respect to the untreated sludge, most likely due to sludge fermentation process in the WWTP.

Regarding the semi-continuous fermentation, microbiological analysis on bacteria (Figure 3A) confirmed the development of different communities in SC.US and SC.HTC. This was not surprising, not only because the characteristics of untreated and HTC-treated sludge were different, but also because untreated sludge, in turn, contained active bacteria that may have played a role in SC.US community evolution, in addition to those coming from the inoculum. Particularly, in SC.US, where untreated sludge was fed, a prevalence of uncharacterized members of phylum Bacteroidota was observed at 42 d (28.4%). Bacteria belonging to this phylum are generally known as saccharolytic, either obtaining sugars from the environment or releasing them from long-chain polysaccharides, such as cellulose or starch (Brown, 2014). Mixed waste sludge is known to be rich in cellulose (Liu et al., 2022), which can therefore be the preferential substrate for these bacteria. The bacterial community observed in SC.US after 42 d does not appear to be simply a sum of those contained in the inoculum and in untreated sludge, but it clearly underwent modifications over time, due to the peculiar conditions maintained in SC.US reactor. Conversely, in SC.HTC, where the thermal pretreatment supposedly generated a higher availability of smaller molecules, a prevalence of Bacillota (previously Firmicutes) was observed, mainly belonging to family Oscillospiraceae (27.5% at 27 d and 19.1% at 56 d), and to genera Limosilactobacillus (17.9% at 27 d and 12.5% at 56 d) and Mitsuokella (2.4% at 27 d and 12.9% at 56 d). A dominance by Firmicutes was previously associated with increased VFA production in mixed culture fermentation reactors (Atasoy et al., 2019). More in detail, Oscillospira and related bacteria are known to be oligosaccharide fermenters, mainly producing butyrate (Gophna et al., 2017). Moreover, the family Oscillospiraceae also contains members, such as Caproicibacter sp. and Caproiciproducens sp., which produce acetate, butyrate, caproate, lactate and ethanol from sugar fermentation, and have been previously isolated from anaerobic digestion reactors (Flaiz et al., 2020; Kim et al., 2015). Limosilactobacillus species, which are common members of the gut microbiota of humans and other vertebrates, are lactic acid bacteria that ferment small sugars (Li et al., 2021), while Mitsuokella is also involved in protein degradation, since it is able to directly ferment several amino acids (Dai et al., 2010), and was previously indicated as one of the main members of bacterial communities in co-fermentation of food wastes and waste sludge (Cheng et al., 2014). The high increase in Mitsuokella abundance in the later stages of SC.HTC might be due to a gradual enrichment in amino acid concentrations in the reactor, which may have been left behind due to preferential fermentation of sugars. However, although the vast majority of bacterial populations observed in SC.HTC generally ferment small molecules, such as sugars and amino acids, a high abundance of the family Lachnospiraceae (12.1% at 27 d and 7.5% at 56 d) was also observed. In fact, members of this family, belonging to phylum Bacillota, are known to collectively ferment a wide variety of complex polysaccharides including glucans, xylans, and pectins (Zaplana et al., 2024). It can be therefore hypothesized that the chosen operational conditions for HTC pretreatment did not allow to reach a complete hydrolysis of all polymers in the sludge. However, as noted above, stronger conditions for HTC treatment, and especially higher temperatures, can lead to toxic compound formation. Thus, the selected conditions appeared to reach an acceptable compromise between opposite requirements, and suitable to promote an efficient acidogenic fermentation for VFA production.

The presence of methanogens in both reactors was confirmed by the characterization of archaeal communities (Figure 3B), although they were partially different (genera Methanobrevibacter and Methanothermobacter in SC.US, genera Methanobrevibacter and Methanosphaera in SC.HTC). They are all hydrogenotrophic methanogens, except for Methanosphaera, which is commonly indicated as methylotrophic (Madigan et al., 2020). This genus was already present also in the inoculum and in untreated sludge, although at low abundances. Apparently, SC.HTC specific conditions exerted a positive selection towards methylotrophic methanogens. It can therefore be hypothesized that HTC pretreatment contributed to an increase in the concentration of solubilized C1 compounds, especially methanol, thus favoring the selection of methylotrophic methanogens besides the hydrogenotrophic ones. Indeed, the presence of methanol in HTCPL from agricultural digestate and cow manure, although HTC pretreatment was performed at 200-220 °C and for a duration of 3 h, has already been reported (Marzban et al., 2023; Urbanowska et al., 2020).

## Biomethane valorization

As introduced in section 2.4.2, BMP1 and BMP2 refer to winter and summer condition, respectively. Results are reported in Figure 4. The BMP1 set was carried out mainly to assess the difference in sludge degradability due to the HTC pretreatment, while second set was performed to have a better understanding of the degradability of the different HTC streams.  Regarding BMP1, it was observed that the net specific biomethane production was higher for the HTCPL (262 ± 3 NmL CH4 g-1 COD) than for the untreated sludge (220.9 ± 6.1 NmL CH4 g-1 COD), indicating a beneficial effect of the thermal hydrolysis on the anaerobic degradability. These results are consistent with what was reported in literature for the same HTC treatment conditions (Ahmed et al., 2021; Danso-Boateng et al., 2015). As for the degradability of the fermented liquor, the SC.HTC and SC.US fermentate had comparable BMP values, resulting in 295 ± 27 NmLCH4 g-1 COD and 267 ± 1 NmLCH4 g-1 COD, respectively.  Comparing the results obtained from HTCPL during the first and second campaign, a different net specific biomethane production was observed, which is likely linked to the seasonality, known to have a significant impact on sewage sludge characteristics (Krzeminski et al., 2012).  The summer sludge (BMP2) appeared to be less anaerobically degradable than the winter sludge. Indeed, the high temperatures during the summer lead to a more effective sludge stabilization within the water line. Overall, it was confirmed that HTCPL has a good anaerobic degradability, despite the pretreatment potentially generating toxic or recalcitrant compounds. Its specific biomethane production is higher than the one of untreated sludge, whether used immediately after pretreatment or downstream of acidogenic fermentation. The BMP2 test was also carried out to compare the BMP of pretreated sludge without solid-liquid separation (BMP2.HTC) with the single fractions of hydrochar and HTCPL (BMP2.Char and BMP2.HTCPL). The test was set-up by adding the same amounts of hydrochar and HTCPL in the HTC bottle as those dosed in bottles with each fraction alone. However, by doing this, the BMP2.HTC reactor was overloaded (F/M value of 0.8 instead of 0.4). Therefore, it is likely that for the first 5 days, the BMP2.HTC bottle experienced acidification, leading to delayed biomethane production. Observing Figure 4B, it can be observed that the specific biomethane production (based on COD) obtained through HTC is higher than that of each single component, suggesting that an increase in load (F/M) helped improving the anaerobic digestion process and/or a synergistic effect of co-digesting the two fractions. As reported in the literature (Ren et al., 2020; Usman et al., 2020; Xu et al., 2023), the electroactivity of hydrochar, i.e. its ability to transfer H+ ions, promotes the anaerobic digestion process. However, electronic interaction occurs only in the presence of microorganisms capable of carrying out this process, and there is no experimental data supporting the hypothesis that the inoculum used contained such microbes. It is important to recall that the hydrochar used during the test results from HTC treatment at 170 °C, a moderate temperature compared to that usually applied in literature for hydrochar production. The specific biomethane production of the BMP2.Char test is very similar to that of the BMP2.HTCPL test, highlighting that hydrochar has a biomethane production capacity fully comparable to that of HTCPL. As a summary, Table 2 reports the anaerobic degradability of the analyzed samples, calculated as the ratio between measured BMP (NmLCH4 g-1 COD) and 315 NmLCH4 g-1 COD, the maximum value achievable from digestion (350 NmL represents the COD value associated with biomethane; 10% of this COD is used for microbial growth, leaving 315 NmL convertible to biomethane). Table 4 provides a comparison among the most recent literature studies regarding methane production from relevant HTC pretreated organic feedstocks. Reported values confirm that BMP obtained by this study well align to the highest values reported in literature for similar HTC treatment conditions.

## Sludge solid-liquid separation

The capillary suction time and filtration time provide a semi-quantitative indication of the dewaterability of a suspension, namely the ease with which water can be removed from the sample under examination. As evident from the values reported in Table 3, both the HTC pretreated sludge and the fermented outlet of the test SC.HTC (semi-continuous fermentation test of HTCPL) exhibited excellent dewatering and sedimentation characteristics compared to the untreated sludge. Thus, the HTC pretreatment significantly improves the sludge dewaterability both in presence and absence of fermentation, making sludge management within a WWTP potentially more efficient and cheaper (Ahmed et al., 2021). Indeed, several studies have shown that heat treatment at temperatures above 150 °C improves the dewaterability of the sludge, usually quantified in terms of a decrease in CST (Bougrier et al., 2008; Fdz-Polanco et al., 2008). In particular, the increase in dewaterability has been attributed to the fact that the thermal treatment acts on the exopolymeric substances and on the cellular structure, favoring the release of the intracellular water and the bound water. This aspect needs to be confirmed through a detailed techno-economic assessment.

## Process assessment

In Figure 5, COD mass balances of the two sludge fermentation alternatives are reported, where overall COD flows are referred to the incoming total COD on the untreated sludge (COD0).  The COD fractionation among particulate COD (pCOD), VFA (sCOD-VFA), and soluble COD components other than VFA (sCOD-other), reflects what has been highlighted in section 3.3: the HTC pretreatment allows obtaining a HTCPL consisting almost exclusively of sCOD, which is converted into VFA in the fermentation reactor according to the acidification efficiency reported above. Conversely, in the SC.US fermenter, a lower fraction of particulate COD is converted into soluble COD, which is then entirely transformed into VFA. Furthermore, it is evident that the fraction lost in gas is almost negligible in all process stages.

The balance across the HTC reactor shows how a significant portion of the COD in the mixed sludge is transferred to the char. This COD fraction does not contribute to VFA production in the fermenter which, as anticipated in section 3.3, is the main cause for a comparable overall acidification yield for the two configurations (with and without HTC pretreatment). For this reason, char valorization is crucial in the perspective of process scale-up. In this sense, considering the obtained BMP value (see section 3.4), char can be used as a feedstock for biomethane production. When comparing the 2 treatment lines, a significant difference in sludge dewaterability must be considered which corresponds to higher costs for fermentate clarification for the SC.US flow. Moreover, the COD-VFA/N-NH4/P-PO4 ratio resulted in 100/6.1/2.4 and 100/8.2/0.9 for SC.US and SC.HTC fermentates, as results of higher protein hydrolysis and phosphorous accumulation in the hydrochar for SC.HTC fermentation. Phosphorus and metals were measure in the untreated sludge and in the HTC products.  Majority of heavy metals were below the detection limits but for Al, Fe, Cu, and Zn. Those metals, together with phosphorus were found to accumulate in the hydrochar  while negligible concentrations remained in the HTCPL (see Supplementary materials). Additional positive side effects may be achieved by applying a thermal pretreatment, including nutrients recovery from HTCPL (namely nitrogen), improved sludge sanitization and removal of contaminants of emerging concern due to thermal oxidation (Zhang et al., 2020). For further developments, additional tests on a broader range of operative conditions exploring a larger combination of temperatures and treatment times would be needed to properly identify optimal conditions of the HTC pretreatment. Moreover, the effect of the feedstock quality, especially as for sludge seasonality, has to be investigated in more details to lead to more robust conclusions

# Conclusions

The present study demonstrates how the HTCPL produced from waste sludge (170 °C, 30 minutes) promotes an efficient acidogenic fermentation for VFA production. The fermented HTCPL hosted different bacterial communities with respect to the fermented sludge without pretreatment, resulting in a fermentate characterized by longer chain VFA, making the fermented liquor valuable for specific downstream applications, such as PHA production. In parallel, hydrochar is a valuable feedstock for biomethane valorization, giving the overall process scheme a promising perspective. Furthermore, the solid-liquid separation of the streams subsequent to the HTC pretreatment shows great improvements compared to the untreated sludge.

E-supplementary data of this work can be found in the online version of the paper.

Acknowledgements

The authors wish to thank Gruppo CAP (Milan, Italy) for providing the sludge samples and for funding the PhD grant of Mr. Matteo Grana. The authors acknowledge Professor Andrea Franzetti and Professor Elena Collina (Università degli studi Milano-Bicocca) for coordinating the microbiological characterization and for the support in the use of HTC device, respectively. Authors are grateful to Ms. Arianna Guerreschi for the support in the experimental work and the analysis performed.

References

Ahmed, M., Andreottola, G., Elagroudy, S., Negm, M.S., Fiori, L., 2021. Coupling hydrothermal carbonization and anaerobic digestion for sewage digestate management: Influence of hydrothermal treatment time on dewaterability and bio-methane production. Journal of Environmental Management 281, 111910. https://doi.org/10.1016/j.jenvman.2020.111910

APHA, 2005. Standard Methods for the Examination of Water and Wastewater, 21st edn. ed. APHA 2005 Standard Methods American Public Health Association/American Water Works Association/Water Environment Federation, Washington DC, USA.

Atasoy, M., Eyice, O., Schnürer, A., Cetecioglu, Z., 2019. Volatile fatty acids production via mixed culture fermentation: Revealing the link between pH, inoculum type and bacterial composition. Bioresource Technology 292, 121889. https://doi.org/10.1016/j.biortech.2019.121889

Boniardi, G., Sessolo, L., Gelmi, E., Turolla, A., Canziani, R., 2024. Targeting phosphorus recovery from sewage sludge while preventing contaminant spread via combined hydrothermal carbonization and wet chemical extraction. Separation and Purification Technology 339, 126620. https://doi.org/10.1016/j.seppur.2024.126620

Bora, R.R., Richardson, R.E., You, F., 2020. Resource recovery and waste-to-energy from wastewater sludge via thermochemical conversion technologies in support of circular economy: a comprehensive review. BMC Chemical Engineering 2, 8. https://doi.org/10.1186/s42480-020-00031-3

Bougrier, C., Delgenès, J.P., Carrère, H., 2008. Effects of thermal treatments on five different waste activated sludge samples solubilisation, physical properties and anaerobic digestion. Chemical Engineering Journal 139, 236–244. https://doi.org/10.1016/j.cej.2007.07.099

Callahan, B.J., McMurdie, P.J., Rosen, M.J., Han, A.W., Johnson, A.J.A., Holmes, S.P., 2016. DADA2: High-resolution sample inference from Illumina amplicon data. Nat Methods 13, 581–583. https://doi.org/10.1038/nmeth.3869

Carolin Christopher, F., Kumar, P.S., Vo, D.-V.N., Joshiba, G.J., 2021. A review on critical assessment of advanced bioreactor options for sustainable hydrogen production. International Journal of Hydrogen Energy 46, 7113–7136. https://doi.org/10.1016/j.ijhydene.2020.11.244

Carrère, H., Bougrier, C., Castets, D., Delgenès, J.P., 2008. Impact of initial biodegradability on sludge anaerobic digestion enhancement by thermal pretreatment. Journal of Environmental Science and Health, Part A 43, 1551–1555. https://doi.org/10.1080/10934520802293735

Castro-Fernandez, A., Taboada-Santos, A., Balboa, S., Lema, J.M., 2023. Thermal hydrolysis pre-treatment has no positive influence on volatile fatty acids production from sewage sludge. Bioresource Technology 376, 128839. https://doi.org/10.1016/j.biortech.2023.128839

Catenacci, A., Azzellino, A., Malpei, F., 2019. Development of statistical predictive models for estimating the methane yield of Italian municipal sludges from chemical composition: a preliminary study. Water Science and Technology 79, 435–447. https://doi.org/10.2166/wst.2019.063

Catenacci, A., Bellucci, M., Yuan, T., Malpei, F., 2020. Chapter 9 - Dairy wastewater treatment using composite membranes, in: Basile, A., Comite, A. (Eds.), Current Trends and Future Developments on (Bio-) Membranes. Elsevier, pp. 261–288. https://doi.org/10.1016/B978-0-12-816823-3.00009-5

Chen, Z., Rao, Y., Usman, M., Chen, H., Białowiec, A., Zhang, S., Luo, G., 2021. Anaerobic fermentation of hydrothermal liquefaction wastewater of dewatered sewage sludge for volatile fatty acids production with focuses on the degradation of organic components and microbial community compositions. Science of The Total Environment 777, 146077. https://doi.org/10.1016/j.scitotenv.2021.146077

Cheng, W., Chen, H., Yan, S., Su, J., 2014. Illumina sequencing-based analyses of bacterial communities during short-chain fatty-acid production from food waste and sewage sludge fermentation at different pH values. World J Microbiol Biotechnol 30, 2387–2395. https://doi.org/10.1007/s11274-014-1664-6

Dai, Z.-L., Zhang, J., Wu, G., Zhu, W.-Y., 2010. Utilization of amino acids by bacteria from the pig small intestine. Amino Acids 39, 1201–1215. https://doi.org/10.1007/s00726-010-0556-9

Danso-Boateng, E., Shama, G., Wheatley, A.D., Martin, S.J., Holdich, R.G., 2015. Hydrothermal carbonisation of sewage sludge: Effect of process conditions on product characteristics and methane production. Bioresource Technology 177, 318–327. https://doi.org/10.1016/j.biortech.2014.11.096

Diaz, R., Mackey, B., Chadalavada, S., kainthola, J., Heck, P., Goel, R., 2022. Enhanced Bio-P removal: Past, present, and future – A comprehensive review. Chemosphere 309, 136518. https://doi.org/10.1016/j.chemosphere.2022.136518

Ding, A., Zhang, R., Ngo, H.H., He, X., Ma, J., Nan, J., Li, G., 2021. Life cycle assessment of sewage sludge treatment and disposal based on nutrient and energy recovery: A review. Science of The Total Environment 769, 144451. https://doi.org/10.1016/j.scitotenv.2020.144451

Du, J., Liu, Y., Zhu, H., 2022. Genome-based analyses of the genus Acidovorax: proposal of the two novel genera Paracidovorax gen. nov., Paenacidovorax gen. nov. and the reclassification of Acidovorax antarcticus as Comamonas antarctica comb. nov. and emended description of the genus Acidovorax. Arch Microbiol 205, 42. https://doi.org/10.1007/s00203-022-03379-7

Dwyer, J., Starrenburg, D., Tait, S., Barr, K., Batstone, D.J., Lant, P., 2008. Decreasing activated sludge thermal hydrolysis temperature reduces product colour, without decreasing degradability. Water Research 42, 4699–4709. https://doi.org/10.1016/j.watres.2008.08.019

Fdz-Polanco, F., Velazquez, R., Perez-Elvira, S.I., Casas, C., del Barrio, D., Cantero, F.J., Fdz-Polanco, M., Rodriguez, P., Panizo, L., Serrat, J., Rouge, P., 2008. Continuous thermal hydrolysis and energy integration in sludge anaerobic digestion plants. Water Science and Technology 57, 1221–1226. https://doi.org/10.2166/wst.2008.072

Ferrentino, R., Langone, M., Gandolfi, I., Bertolini, V., Franzetti, A., Andreottola, G., 2016. Shift in microbial community structure of anaerobic side-stream reactor in response to changes to anaerobic solid retention time and sludge interchange ratio. Bioresource Technology 221, 588–597. https://doi.org/10.1016/j.biortech.2016.09.077

Ferrentino, R., Sacchi, G., Scrinzi, D., Andreottola, G., Fiori, L., 2023. Valorization of swine manure for a circular approach through hydrothermal carbonization. Biomass and Bioenergy 168, 106689. https://doi.org/10.1016/j.biombioe.2022.106689

Flaiz, M., Baur, T., Brahner, S., Poehlein, A., Daniel, R., Bengelsdorf, F.R., 2020. Caproicibacter fermentans gen. nov., sp. nov., a new caproate-producing bacterium and emended description of the genus Caproiciproducens. International Journal of Systematic and Evolutionary Microbiology 70, 4269–4279. https://doi.org/10.1099/ijsem.0.004283

Garlapalli, R.K., Wirth, B., Reza, M.T., 2016. Pyrolysis of hydrochar from digestate: Effect of hydrothermal carbonization and pyrolysis temperatures on pyrochar formation. Bioresource Technology 220, 168–174. https://doi.org/10.1016/j.biortech.2016.08.071

Gophna, U., Konikoff, T., Nielsen, H.B., 2017. Oscillospira and related bacteria - From metagenomic species to metabolic features. Environ Microbiol 19, 835–841. https://doi.org/10.1111/1462-2920.13658

Grana, M., Catenacci, A., Ficara, E., 2024. Denitrification Capacity of Volatile Fatty Acids from Sludge Fermentation: Lab-Scale Testing and Full-Scale Assessment. Fermentation 10, 25. https://doi.org/10.3390/fermentation10010025

Hitch, T.C.A., Bisdorf, K., Afrizal, A., Riedel, T., Overmann, J., Strowig, T., Clavel, T., 2022. A taxonomic note on the genus Prevotella: Description of four novel genera and emended description of the genera Hallella and Xylanibacter. Systematic and Applied Microbiology 45, 126354. https://doi.org/10.1016/j.syapm.2022.126354

James W. Brown, 2014. Principles of Microbial Diversity. ASM Press.

Kacprzak, M., Neczaj, E., Fijałkowski, K., Grobelak, A., Grosser, A., Worwag, M., Rorat, A., Brattebo, H., Almås, Å., Singh, B.R., 2017. Sewage sludge disposal strategies for sustainable development. Environmental Research 156, 39–46. https://doi.org/10.1016/j.envres.2017.03.010

Kim, B.-C., Seung Jeon, B., Kim, S., Kim, H., Um, Y., Sang, B.-I., 2015. Caproiciproducens galactitolivorans gen. nov., sp. nov., a bacterium capable of producing caproic acid from galactitol, isolated from a wastewater treatment plant. International Journal of Systematic and Evolutionary Microbiology 65, 4902–4908. https://doi.org/10.1099/ijsem.0.000665

Krzeminski, P., Iglesias-Obelleiro, A., Madebo, G., Garrido, J.M., van der Graaf, J.H.J.M., van Lier, J.B., 2012. Impact of temperature on raw wastewater composition and activated sludge filterability in full-scale MBR systems for municipal sewage treatment. Journal of Membrane Science 423–424, 348–361. https://doi.org/10.1016/j.memsci.2012.08.032

Li, F., Cheng, C.C., Zheng, J., Liu, J., Quevedo, R.M., Li, J., Roos, S., Gänzle, M.G., Walter, J., 2021. Limosilactobacillus balticus sp. nov., Limosilactobacillus agrestis sp. nov., Limosilactobacillus albertensis sp. nov., Limosilactobacillus rudii sp. nov. and Limosilactobacillus fastidiosus sp. nov., five novel Limosilactobacillus species isolated from the vertebrate gastrointestinal tract, and proposal of six subspecies of Limosilactobacillus reuteri adapted to the gastrointestinal tract of specific vertebrate hosts. International Journal of Systematic and Evolutionary Microbiology 71, 004644. https://doi.org/10.1099/ijsem.0.004644

Liang, T., Elmaadawy, K., Liu, B., Hu, J., Hou, H., Yang, J., 2021. Anaerobic fermentation of waste activated sludge for volatile fatty acid production: Recent updates of pretreatment methods and the potential effect of humic and nutrients substances. Process Safety and Environmental Protection 145, 321–339. https://doi.org/10.1016/j.psep.2020.08.010

Libra, J.A., Ro, K.S., Kammann, C., Funke, A., Berge, N.D., Neubauer, Y., Titirici, M.-M., Fühner, C., Bens, O., Kern, J., Emmerich, K.-H., 2011. Hydrothermal carbonization of biomass residuals: a comparative review of the chemistry, processes and applications of wet and dry pyrolysis. Biofuels 2, 71–106. https://doi.org/10.4155/bfs.10.81

Liu, R., Li, Y., Zhang, M., Hao, X., Liu, J., 2022. Review on the fate and recovery of cellulose in wastewater treatment. Resources, Conservation and Recycling 184, 106354. https://doi.org/10.1016/j.resconrec.2022.106354

Lucian, M., Volpe, M., Merzari, F., Wüst, D., Kruse, A., Andreottola, G., Fiori, L., 2020. Hydrothermal carbonization coupled with anaerobic digestion for the valorization of the organic fraction of municipal solid waste. Bioresource Technology 314, 123734. https://doi.org/10.1016/j.biortech.2020.123734

Madigan MT, Bender KS, Buckley DH, Sattley WM, Stahl DA, 2020. Brock Biology of Microorganisms, Global Edition, 16th edition, 16th ed. Pearson Ed.

Malhotra, M., Garg, A., 2023. Hydrothermal carbonization of sewage sludge: Optimization of operating conditions using design of experiment approach and evaluation of resource recovery potential. Journal of Environmental Chemical Engineering 11, 109507. https://doi.org/10.1016/j.jece.2023.109507

Mantovani, M., Collina, E., Marazzi, F., Lasagni, M., Mezzanotte, V., 2022. Microalgal treatment of the effluent from the hydrothermal carbonization of microalgal biomass. Journal of Water Process Engineering 49, 102976. https://doi.org/10.1016/j.jwpe.2022.102976

Maragkaki, A.E., Vasileiadis, I., Fountoulakis, M., Kyriakou, A., Lasaridi, K., Manios, T., 2018. Improving biogas production from anaerobic co-digestion of sewage sludge with a thermal dried mixture of food waste, cheese whey and olive mill wastewater. Waste Management 71, 644–651. https://doi.org/10.1016/j.wasman.2017.08.016

Marreiros, B.C., Carvalheira, M., Henriques, C., Pequito, D., Nguyen, Y., Solstad, R.G., Eksteen, J.J., Reis, M.A.M., 2023. Pilot-scale valorisation of salmon peptone into polyhydroxyalkanoates by mixed microbial cultures under conditions of high ammonia concentration. Journal of Environmental Chemical Engineering 11, 110100. https://doi.org/10.1016/j.jece.2023.110100

Marzban, N., Libra, J.A., Rotter, V.S., Ro, K.S., Moloeznik Paniagua, D., Filonenko, S., 2023. Changes in Selected Organic and Inorganic Compounds in the Hydrothermal Carbonization Process Liquid While in Storage. ACS Omega 8, 4234–4243. https://doi.org/10.1021/acsomega.2c07419

Matos, M., Cruz, R.A.P., Cardoso, P., Silva, F., Freitas, E.B., Carvalho, G., Reis, M.A.M., 2021. Combined Strategies to Boost Polyhydroxyalkanoate Production from Fruit Waste in a Three-Stage Pilot Plant. ACS Sustainable Chem. Eng. 9, 8270–8279. https://doi.org/10.1021/acssuschemeng.1c02432

Merzari, F., Goldfarb, J., Andreottola, G., Mimmo, T., Volpe, M., Fiori, L., 2020. Hydrothermal Carbonization as a Strategy for Sewage Sludge Management: Influence of Process Withdrawal Point on Hydrochar Properties. Energies 13, 2890. https://doi.org/10.3390/en13112890

Mottet, A., Steyer, J.P., Déléris, S., Vedrenne, F., Chauzy, J., Carrère, H., 2009. Kinetics of thermophilic batch anaerobic digestion of thermal hydrolysed waste activated sludge. Biochemical Engineering Journal 46, 169–175. https://doi.org/10.1016/j.bej.2009.05.003

Ren, S., Usman, M., Tsang, D.C.W., O-Thong, S., Angelidaki, I., Zhu, X., Zhang, S., Luo, G., 2020. Hydrochar-Facilitated Anaerobic Digestion: Evidence for Direct Interspecies Electron Transfer Mediated through Surface Oxygen-Containing Functional Groups. Environ. Sci. Technol. 54, 5755–5766. https://doi.org/10.1021/acs.est.0c00112

Suryawan, I.W.K., Septiariva, I.Y., Widanarko, D.U.F., Qonitan, F.D., Sarwono, A., Sari, M.M., Prayogo, W., Arifianingsih, N.N., Suhardono, S., Lim, J.-W., 2024. Enhancing energy recovery from Wastewater Treatment Plant sludge through carbonization. Energy Nexus 14, 100290. https://doi.org/10.1016/j.nexus.2024.100290

Urbanowska, A., Kabsch-Korbutowicz, M., Wnukowski, M., Seruga, P., Baranowski, M., Pawlak-Kruczek, H., Serafin-Tkaczuk, M., Krochmalny, K., Niedzwiecki, L., 2020. Treatment of Liquid By-Products of Hydrothermal Carbonization (HTC) of Agricultural Digestate Using Membrane Separation. Energies 13, 262. https://doi.org/10.3390/en13010262

Usman, M., Shi, Z., Ren, S., Ngo, H.H., Luo, G., Zhang, S., 2020. Hydrochar promoted anaerobic digestion of hydrothermal liquefaction wastewater: Focusing on the organic degradation and microbial community. Chemical Engineering Journal 399, 125766. https://doi.org/10.1016/j.cej.2020.125766

Wang, L., Li, A., 2015. Hydrothermal treatment coupled with mechanical expression at increased temperature for excess sludge dewatering: The dewatering performance and the characteristics of products. Water Research 68, 291–303. https://doi.org/10.1016/j.watres.2014.10.016

Weber, B., Ayala-Mercado, I.D., Stadlbauer, E.A., 2024. Steam explosion versus hydrothermal carbonization: evaluation of applicability for pretreatment of semi-solid waste from beverage industries to facilitate on-site biogas production. Biomass Conv. Bioref. 14, 7659–7671. https://doi.org/10.1007/s13399-022-02983-3

Willems, A., 2014. The Family Comamonadaceae, in: Rosenberg, E., DeLong, E.F., Lory, S., Stackebrandt, E., Thompson, F. (Eds.), The Prokaryotes: Alphaproteobacteria and Betaproteobacteria. Springer, Berlin, Heidelberg, pp. 777–851. https://doi.org/10.1007/978-3-642-30197-1_238

Xu, Q., Yang, G., Liu, X., Wong, J.W.C., Zhao, J., 2023. Hydrochar mediated anaerobic digestion of bio-wastes: Advances, mechanisms and perspectives. Science of The Total Environment 884, 163829. https://doi.org/10.1016/j.scitotenv.2023.163829

Zaplana, T., Miele, S., Tolonen, A.C., 2024. Lachnospiraceae are emerging industrial biocatalysts and biotherapeutics. Front. Bioeng. Biotechnol. 11. https://doi.org/10.3389/fbioe.2023.1324396

Zhang, X., Li, X., Li, R., Wu, Y., 2020. Hydrothermal Carbonization and Liquefaction of Sludge for Harmless and Resource Purposes: A Review. Energy Fuels 34, 13268–13290. https://doi.org/10.1021/acs.energyfuels.0c02467

Table captions

Table 1 Effect of HTC operational parameters on sludge solubilization in HTCPL. HTC time 0 refers to a sample taken right after the target temperature was achieved.

Table 2 Anaerobic degradability of the different samples involved in this study, assessed as the ratio of their obtained BMP and the maximum theoretical value. N/A stands for Not Available.

Table 3 Dewaterability assessment of untreated sludge (US), HTC pretreated sludge (HTC), and pretreated fermented sludge (SC.HTC). N/A stands for Not Available.

Table 4 Biomethane production from relevant HTC-pretreated organic feedstocks (prior to biological treatments) in recent works. OFMSW indicates Organic Fraction of Municipal Solid Waste, BSG indicates Brewery Spent Grains, AB indicates Agave Bagasse. * Approximate values obtained from graphs.

Figure captions

Figure 1 Trend of concentrations for (A) sCOD and VFA, and (B) ammonium and phosphate in the effluent of SC.US and SC.HTC. The SC.US fermenter was started after the end of the acclimatization period of SC.HTC. The latter was performed keeping the HRT three times higher than the target for the following continuous operation.

Figure 2 VFA average composition (in terms of COD) for SC.HTC and SC.US tests.

Figure 3 Bacterial community structure at genus level (A). Whenever a classification at genus level was not achieved, bacterial populations were reported as ‘unclassified’ and referred to the lowest taxonomic level that was classified (confidence = 0.8). All genera that did not reach a relative abundance of at least 2% in at least one sample were grouped in ‘Other genera’. Archaeal community structure at genus level (B). All genera that did not reach a relative abundance of at least 1% in at least one sample were grouped in ‘Other genera’. FS-inoculum: fermented sludge used as inoculum; US: untreated sludge fed to SC.US at time = 0 d; SC.US-42d: fermentate from SC.US collected at time = 42 d; SC.HTC-27d and SC.HTC-56d: fermentate from SC.HTC collected at time = 27 and 56 d, respectively.

Figure 4 BMP1 (A) and BMP2 (B) tests.

Figure 5 COD mass balances of SC.HTC (A) and SC.US (B) tests. Solid/liquid separation after HTC pretreatment is reported (S/L sep.). 100% COD0 represents the total COD entering in the system as US, sCOD other indicates the sCOD that is not VFAs, while COD-part. indicates the particulate COD.

Tables

Table 1 Effect of HTC operational parameters on sludge solubilization in HTCPL. HTC time 0 refers to a sample taken right after the target temperature was achieved.

| HTC temperature [°C] | HTC time [min] | sCOD [g L-1] | sCOD specific release [mg sCOD g-1 COD,in] | VFA [g COD L-1] | VFA specific release [mg COD g-1 COD,in] |
| --- | --- | --- | --- | --- | --- |
| 170 | 0 | 6.5 | 76 | 1.0 | 15 |
|  | 30 | 8.0 | 133 | 1.6 | 36 |
|  | 60 | 9.5 | 191 | 1.3 | 26 |
|  | 150 | 11.7 | 275 | 1.2 | 22 |
| 190 | 30 | 7.3 | 107 | 1.1 | 16 |
|  | 60 | 8.0 | 133 | 1.1 | 15 |
|  | 150 | 8.5 | 152 | 0.9 | 8 |

Table 2 Anaerobic degradability of the different samples involved in this study, assessed as the ratio of their obtained BMP and the maximum theoretical value. N/A stands for Not Available.

| Sample | Anaerobic degradability [%] |  |
| --- | --- | --- |
|  | BMP1 | BMP2 |
| US | 70 | N/A |
| HTC | N/A | 86 |
| HTCPL | 83 | 66 |
| SC.US out | 85 | N/A |
| SC.HTC out | 94 | N/A |
| HTC.Char | N.D | 61 |

Table 3 Dewaterability assessment of untreated sludge (US), HTC pretreated sludge (HTC), and pretreated fermented sludge (SC.HTC). N/A stands for Not Available.

| Sample | TS [g L-1] | CST [s] | TTF [min] | SVI [mL g-1 TS] |
| --- | --- | --- | --- | --- |
| US | 41 | 338 ± 6 | 150 | 24.3 |
| HTC | 28 | 28 ± 5 | 1.5 | 18.1 |
| SC.HTC | 3.1 | 20 ± 1 | N/A | N/A |

Table 4 Biomethane production from relevant HTC-pretreated organic feedstocks (prior to biological treatments) in recent works. OFMSW indicates Organic Fraction of Municipal Solid Waste, BSG indicates Brewery Spent Grains, AB indicates Agave Bagasse. * Approximate values obtained from graphs.

| Feedstock | HTC temperature & process time | Digested fraction | COD reduction | BMP | Reference |
| --- | --- | --- | --- | --- | --- |
| SS | 190 °C, 0.5h | HTCPL | 35% | 142±3 SmL CH4 g -1 COD | (Ahmed et al., 2021) |
|  | 190 °C, 1h |  | 37% | 103±9 SmL CH4 g -1 COD |  |
|  | 190 °C, 2h |  | 45% | 92±17 SmL CH4 g -1 COD NmS CH4 g -1 COD |  |
|  | 190 °C, 3h |  | 44% | 84±1 SmL CH4 g -1 COD |  |
| SS | 200 °C, 1h | HTCPL | N.A. | 220 mL CH4 g -1 COD | (Malhotra and Garg, 2023) |
| OFMSW | 180 °C, 1h | HTC | 40%* | 350 SmL CH4 g -1 COD | (Lucian et al., 2020) |
|  |  | HTCPL | 38%* | 205 SmL CH4 g -1 COD * |  |
|  | 220 °C, 3h | HTC | 45%* | 150 SmL CH4 g -1 COD * |  |
|  |  | HTCPL | 45%* | 185 SmL CH4 g -1 COD * |  |
|  | 250 °C, 6h | HTC | 55%* | 70 SmL CH4 g -1 COD * |  |
|  |  | HTCPL | 42%* | 166 SmL CH4 g -1 COD |  |
| Swine manure |  | 190 °C, 1h | HTC | N.A. | 299±4 SmL CH4 g -1 VS | (Ferrentino et al., 2023) |
|  |  |  | HTC.Char | N.A. | 340±40 SmL CH4 g -1 VS |  |
| BSG | 200 °C, 3h | HTCPL | N.A. | 265 NmL CH4 g -1 COD * | (Weber et al., 2024) |
| AB | 200 °C, 3h | HTCPL | N.A. | 245 NmL CH4 g -1 COD * |  |
| SS | 170 °C, 0.5h | HTC | 76% | 271±2 NmL CH4 g -1 COD | This study |
|  | 170 °C, 0.5h | HTC.Char | 51% | 191±1 NmL CH4 g -1 COD |  |
|  | 170 °C, 0.5h | HTCPL | 75% | 236±30 NmL CH4 g -1 COD |  |

Figures

Figure 1 Fermentate characteristics in the semi-continuous fermentation test: (A) sCOD and VFA, and (B) ammonium and phosphate. The SC.US fermenter was started after the acclimatization period of SC.HTC.

Figure 2 VFA average composition (in terms of COD) for SC.HTC and SC.US tests.

(A)

(B)

Figure 3 Bacterial community structure at genus level (A). Whenever a classification at genus level was not achieved, bacterial populations were reported as ‘unclassified’ and referred to the lowest taxonomic level that was classified (confidence = 0.8). All genera that did not reach a relative abundance of at least 2% in at least one sample were grouped in ‘Other genera’. Archaeal community structure at genus level (B). All genera that did not reach a relative abundance of at least 1% in at least one sample were grouped in ‘Other genera’. FS-inoculum: fermented sludge used as inoculum; US: untreated sludge fed to SC.US at time = 0 d; SC.US-42d: fermentate from SC.US collected at time = 42 d; SC.HTC-27d and SC.HTC-56d: fermentate from SC.HTC collected at time = 27 and 56 d, respectively.

Figure 4 BMP1 (A) and BMP2 (B) tests.

Figure 5 COD mass balances of SC.HTC (A) and SC.US (B) tests. Solid/liquid separation after HTC pretreatment is reported (S/L sep.). 100% COD0 represents the total COD entering in the system as US, sCOD-other indicates the sCOD that is not VFAs, while COD-part is COD associated to particulate components.